M12 and 38999 Connector, Wiring, and Signal Guide

Contents

1.	M12 Connector Types and Pinouts	2

2.	MIL-C-38999 BCM side connector pinouts	3

3.	Up-trunk M12 connector signals:	5

4.	Down-Trunk M12 Connector Signals:	6

5.	Power-Trunk M12 Connector Signals:	6

6.	UI Touchscreen Display	7

7.	Probing adapters	8

- M12 Connector Types and Pinouts

We use the following types of M12 connectors

M12, 4 pin, type A   (Male and Female)

M12, 4 pin, type D  (Male and Female)

M12, 5 pin, type A  (Male and Female)

M12, 8 pin, type A  (Male and Female)

- MIL-C-38999 BCM side connector pinouts

| D38999 Harness | Upper Trunk | Down Trunk |
| --- | --- | --- |
| Brain Part Number | 215-00139-01 | 215-00169-01 |
| Brain Part Name | 38999, 66-Pin, N, M, D38999/26FF35PN | 38999, 66-Pin, A, M, D38999/26FF35PA |
| Manufacturer | Souriau | Souriau |
| Manufacturer PN | D38999/26FF35PN | D38999/26FF35PA |
| Shell Size | F | F |
| Contact Layout | 35 | 35 |
| Number of Contacts | 66 | 66 |
| Where-used Assy | 810-00200-XX 810-00212-01 810-00219-01 | 810-00129-XX 810-00209-01 810-00221-01 |
| Figure No. | 1 | 1 |

Figure No. 1: Front Face of pin insert

| D38999 Harness | Power Trunk |
| --- | --- |
| Brain Part Number | 215-00170-01 |
| Brain Part Name | 38999, 8-Pin, N, F, 8D5-13F26SN |
| Manufacturer | Souriau |
| Manufacturer PN | 8D5-13F26SN |
| Shell Size | 13 |
| Contact Layout | 26 |
| Number of Contacts | 8 |
| Where-used Assy | 810-00132-XX 810-00213-01 |
| Figure No. | 2 |

Figure No. 2: Front face of pin insert

| D38999 Harness | Display |
| --- | --- |
| Brain Part Number | 215-00171-01 |
| Brain Part Name | Housing, D38999, 22-Pin, N, M, D38999/26FC35PN |
| Manufacturer | Souriau |
| Manufacturer PN | D38999/26FC35PN |
| Shell Size | C |
| Contact Layout | 35 |
| Number of Contacts | 22 |
| Where-used Assy | 810-00130-XX 810-00223-XX |
| Figure No. | 3 |

Figure No. 3: Front face of pin insert

The following signals are carried on M12 connectors.  See the specific Up trunk 810-00173-01, down trunk 810-00129-01, and power trunk 810-00132-01 drawings for more details

- Up-trunk M12 connector signals:

Up-trunk Drawing P/N 810-00173-01

“CAM” are the RGB cameras (USB)

This end column shows the M12 connector  type A or D , number of pins, and male or female

- Down-Trunk M12 Connector Signals:

Down-trunk drawing P/N 810-00129-01

ENC_ABS is the steering encoder AKA absolute encoder

- Power-Trunk M12 Connector Signals:

Power-Trunk Drawing P/N 810-00132-01

- UI Touchscreen Display

UI Touchscreen Display Cable P/N 810-00130-01

Video Display  (LVDS)

Backlight

Touch Detect  (USB)

- Probing adapters

These adapters are useful to help probe the connectors.  They are available on amazon at

https://www.amazon.com/EDGELEC-Breadboard-Optional-Assorted-Multicolored/dp/B07GD2BWPY